Hardware Documentation:

Power Source: High-Capacity Li-ion/LiFePO4 Pack

The robot is powered by a central high-energy density battery pack designed for

extended runtimes and high current discharge.

• Nominal Voltage: 12V

• Capacity: (6000 MAH)

• Calculated Energy: 72WH

• Application Note: This capacity allows for approximately 1.5 to 3 hours of

continuous operation, depending on the load on the 4x DC motors, the

MG996R, & THE s3003 Servos activity.

Power Distribution & Regulation

The system utilizes a dual-rail power architecture to isolate "noisy" motor

power from the "sensitive" logic power of the ESP32.

12V High-Power Rail (Unregulated)

• Feeds: 2x L298N Motor Drivers.

• Purpose: Provides raw voltage to the 250 RPM DC motors to maximize torque

(8.8 kgf.cm).

5V Logic & Servo Rail (Regulated)

• Component: High-Efficiency DC-DC Buck Converter (XL4005).

• Input: 12V | Output: 5V (Adjusted).

• Feeds: ESP32 (38-pin DIP), MG996R Servo, and S3003 Servo.

• Design Note: The Buck converter must be rated for at least 3A to 5A to

handle the stall current of the MG996R (2.5A) without dropping voltage to

the ESP32.

Control & Logic: ESP32 (38-Pin DIP)

The ESP32 acts as the "brain," managing WiFi/Bluetooth connectivity and PWM

(Pulse Width Modulation) signals for speed and position control.

• Pin Configuration: Utilizing the 38-pin layout for maximum GPIO

availability.

• PWM Channels: * 4 Channels for DC Motor Speed (L298N Enable Pins).

o 2 Channels for Servo Position (MG996R & S3003).

• Logic Level Note: The ESP32 operates at 3.3V. While the L298N and Servos can

often trigger at 3.3V, a logic level shifter may be considered if signal

instability occurs.

Motor Actuation: Dual L298N Drivers

To handle four DC motors, two L298N H-Bridge modules are used in a "Skid-Steer"

configuration.

Driver Channel Motor Position

L298N #1 A / B

Control Pins

Front-Left / Rear-Left ENA, IN1, IN2, IN3, IN4, ENB

L298N #2 A / B

Export to Sheets

Front-Right / Rear-Right ENA, IN1, IN2, IN3, IN4, ENB

• Heat Dissipation: Due to the 1mm steel chassis, the L298N heat sinks can be

thermally coupled to the chassis to act as a secondary heat spreader.

• Voltage Drop: Note that the L298N has an internal voltage drop of about

1.5V to 2V. Your motors will effectively see ~10V from the 12V battery.

Wiring & Safety

• Common Ground: All components (Battery, Buck Converter, L298Ns, and

ESP32) must share a Common Ground (GND) to ensure signal integrity.

• Fuse Protection: A 10A Inline Fuse is recommended between the battery

and the 12V rail to protect the 6000mAh pack from accidental shorts

against the steel body.

• Gauge: Use 18AWG wire for the 12V motor lines and 22AWG for the 5V logic

lines.