Mechanical Documentation:

Executive Summary

This report outlines the mechanical architecture of a mobile robotic platform.

The design prioritizes structural rigidity through a 1mm galvanized steel

chassis, high-torque mobility via a quad-DC motor drivetrain, and a 2-Degree of

Freedom (DOF) manipulative arm.

Drivetrain & Locomotion

The robot utilizes a 4-wheel drive (4WD) configuration to ensure maximum

traction and payload capacity.

Component

Specification

Actuators

Rated Speed

Stall Torque

Wheels

4x DC Motors

250 RPM

8.8kgf.cm

[65mm]

Structural Framework

Function

Primary propulsion and skid-steering.

Balanced for indoor/outdoor

traversal.

High torque for overcoming structural

weight.

Directly coupled to motor shafts.

The chassis is the backbone of the robot, engineered to protect internal

electronics while providing mounting points for the manipulator.

• Material: 1mm Galvanized Steel sheet

• Fabrication Method: Plasma cutting followed by sheet metal

bending.Rigidity: The 1mm gauge offers high torsional stiffness, preventing

"frame twist" during high-torque maneuvers.

• Assembly: Fastened using M3/M4 stainless steel bolts, nuts, & Washers to

withstand motor vibrations.

Manipulator Design (Robot Arm)

The arm is designed as a modular attachment positioned at the [Front/Center] of

the chassis.

Base Rotation (Azimuth)

• Actuator: MG996R High-Torque Metal Gear Servo.

• Design Note: The metal gears are essential here to handle the radial

loads and momentum of the arm's swing.

• Range of Motion: 360° horizontal sweep.

End-Effector (Gripper)

• Actuator: S3003 Standard Servo.

• Mechanism: Parallel gripper jaws with a slight angle for better grip due

to slack in griper links

• Design Note: The S3003 provides precise, lightweight control for the jaws,

where high torque is less critical than speed and weight savings at the

"fingertips.

Technical Calculations (Estimates)

To ensure the motors can handle the steel body, we use the following relation

for torque (t) required per motor:

• = Total mass (Steel chassis + Motors + Battery + Electronic Components).

• = Wheel radius.

• = Desired acceleration.

• = Drivetrain efficiency.

Given the rating, this platform is optimized for a total weight of approximately

[~4.7 kg] while maintaining 20 degree incline capabilities.

Maintenance & Safety

• Sheet Metal Safety: All exposed 1mm steel edges are filleted to prevent

wire chafing and to provide safe handling.