Software Documentation:

System Overview & Control Modes

The robot operates as a dual-mode mobile manipulator. A physical hardware

switch toggles the ESP32 between two distinct states:

• Mode A: Autonomous Line Following: Uses a 3-channel IR sensor array to

navigate pre-marked paths.

• Mode B: Manual Remote Control: Uses Bluetooth (HID) to interface with a

PS4 controller for 4WD movement and 2-DOF arm manipulation.

Drivetrain & Locomotion Logic

The robot utilizes a 4-Wheel Drive (4WD) Tank Drive configuration. For

simplified control, the motors are paired (Left Front/Rear and Right

Front/Rear).

• Actuation: 4 DC Motors.

• Speed Control: 8-bit PWM ( to ) via ESP32 LEDC or PWM pins.

• Directional Control: 2 Digital Pins per motor pair (H-Bridge logic).

• Movement Profiles: * Forward/Backward: Both sides sync.

o Pivot Turn: One side slows down (soft curve).

o Spin-in-Place: Sides rotate in opposite directions.

Autonomous Logic: Line Following

The system processes feedback from three Active LOW IR Sensors (Left, Center,

Right).

Sensor State Physical

Meaning

Middle = LOW Line

centered

Left = LOW

Veering

Right

Robot Action

Move Forward

Soft Left Curve (Adjust speed)

Right = LOW Veering Left Soft Right Curve (Adjust speed)

Two Sensors

= LOW

All Sensors

= HIGH

Sharp Turn Strong Spin Correction

Line Lost

Memory Logic: Spins in the last known direction for

a set duration; stops if the line is not re-acquired.

Manual Logic: PS4 Remote Control

The ESP32 establishes a Bluetooth connection to a PS4 controller.

• Joystick Mapping: The Left Joystick values (X and Y axes) are mixed to

calculate differential steering speeds for the left and right motor banks.

• Deadzone Management: A software threshold is applied to joystick inputs

to ignore "stick drift" or accidental micro-movements when the sticks are

centered.

• Arm Manipulation:

o MG996R (Arm Rotation): Controlled via specific buttons (e.g., L1/R1 or

D-Pad) with software-defined angle limits to prevent mechanical

strain on the 1mm steel frame.

o S3003 (Gripper): Toggle or button-hold logic to open and close the

jaws.

Software Architecture

The code is modularized into functional blocks to ensure high performance and

easy debugging:

• motor_drive(): Low-level PWM and GPIO pin management.

• line_logic(): Interprets IR sensor data and executes recovery maneuvers.

• remote_handler(): Processes Bluetooth packets and maps PS4 inputs to

motor/servo variables.

• safety_check(): Enforces servo angle limits and "Line Lost" timeouts.

Bill Of MATERIAL:

• 4x JGA25-370 250RPM 8.8KGF.CM (~1200 LE)

• 1X MG996R Servo motor (Continuous Rotation) (195 LE)

• 1x S3003 Servo motor (Continuous Rotation) (150 LE)

• 4x Robot wheel + couplers (65mm x 25mm) (450 LE)

• 1x Battery Pack 12v 6000mah (900 LE)

• 1x Esp32 38pins dip (350 LE)

• 2x L298N MOTOR DRIVE (130 LE)

• 1X XL4005 BUCK CONVERTER (80 LE)

• 1X Gripper (PLA / Petg) (300 LE)

• 3x LM393 IR SENSOR (90 LE)

• Wires / connecters / Terminal Blocks (100 LE)

• Electronic Components (Mosfet Regulators / Diodes / capacitors / Resistors

/ Switches) (50 LE)

• 1x pcb (100 LE)

• 1x Sheet metal body (550 LE)

• Cooling Fan (40 le)

• Etching acid (90 le)

• Plastic Spacers (15 le)